# Design changes for 2025 MCHA system

Table of Contents

Design changes for 2025 MCHA system	1

1.	Overarching Changes	2

2.	PEN	2

3.	DEN	2

4.	CEN	3

5.	HIP	4

6.	LVD	4

7.	AIL	5

8.	Expansion Board	5

## Overarching Changes

- Shutdown circuit traces

- Keyed connectors where possible

## PEN

- Same

  - Connector pinout

  - Majority of circuitry

  - Functionality of board

- Changes

  - Zeners removed on analogue, low voltage signals on step down circuits for teensy

  - Shape and mounting location

  - Fuses on sensor inputs

    - 0.1 A

      - Chosen from powering sensors with power supply, there was no current seen (at a an accuracy level of 0.00 A), so this value was chosen

  - 2mm shutdown traces

  - Low pass filters on brake signal inputs

  - LED on BOTs on teensy step down

## DEN

- Same

  - Everything

- Changes

  - Nothing

## CEN

- Same

  - DT pinouts

  - Teensy step downs

- Changes

  - Interpose relay (from LVD to CEN)

  - Shape and mounting location of PCB

  - MOSFETs for cooling, brake light and sounder switching

  - Fuse sizes for cooling, brake light and sounder

  - Teensy pinout

  - 2mm shutdown traces

  - Keyed AT connectors for all 8 pins, and 6 pin connectors

- BSPD

  - Same

    - Functionality of the board

  - Changes

    - Pot for current_sense

    - 1206 capacitor on current_sense filter

- Recommendations

- Use multi-turn potentiometer for tuning, surface mount better for vibrations tho

- Fix back silkscreen

- HFR

  - Same

    - Everything

  - Changes

    - nothing

## HIP

- Same

  - DT pinouts

  - Fill area for 12V_RAW

  - Overall functionality of the PCB

- Changes

  - EM and DCDC are HV rated connectors

  - MC, Accu, DCDC, EM all have interlocks

  - New master switches

  - HVD naming to MSD

  - Micro fit molex to Mini fit molex for HV sense connections

  - Component layout

- TSAL_Discharge

  - Same

    - All of circuitry

  - Changes

    - Red

    - Move bolt hole for discharge resistor heatsink

    - Traces for switched red and green

- Recommendations

- Thermistor placement

## LVD

- Same

  - DT mounting position

  - DT pinouts

  - Fits within lid constraints

- Changes

  - Interlock circuit

    - Teensy pinout

    - Teensy code

    - DBC and MoTeC updates

  - 22 pin molex split into 3 molex connectors

  - Fan switching circuit

    - Relay to 2 sets of switching mosfets

  - Precharge, BMS and IMD fuse sizes

  - Teensy pinout

  - Fan control pins

  - Remove interpose circuit

## AIL

- Same

  - Turn on a light above 60 V

- Changes

  - Mounting

  - Efficient circuitry

  - Works

## Expansion Board

- Same

  - Logging sensor data

- Changes

  - fixes to CAN lines and GND plane

  - Shock pots (have)

  - wheel speed sensors (order 3 more)

  - steering angle sensor (have)

  - SWEN functionality

    - dials for torque

    - switch for regen

    - idk how you'll make sure these things are in the correct position after unplugging the board from the car

      - Programmable button

  - 3.3v regulator

  - LCD screen

  - temp sensors for radiator

    - temp sensors are already in the radiator

  - DBC file update

  - MoTeC update