ZERO-TO-HERO

How To Import PCBs Into OnShape

Jacob Lukes

2025

Contents

Sanity Checks	2

How To Import PCB Into OnShape	2

How To Import PCBs Into an Assembly	8

# Sanity Checks

- Firstly, ensure ALL step files for footprints are present and correctly positioned

  - This includes:

    - All dimensions for components are correct, mainly large components that protrude from the board. It is the PCB designer’s responsibility to know this

    - Checking that mating connectors and attached wires are accounted for

      - eg: Micro/Mini Fit MOLEX connectors that attach to the board in any way

      - This can be as simple as editing the original STEP file for the connector, and extruding it

  - Getting this part right saves LOTS of time down the line for the individual making the enclosure and the team as a whole

  - Refer to PCB Design Zero-To-Hero on how to make and attach STEP files to footprints

# How To Import PCB Into OnShape

- From within KiCad, export the PCB as a STEP file

- Open OnShape, ensure you are in the correct NU Racing Project Folder for the given year (eg: NU-25)

  - You may need to gain access from the Mechanical Department Leader first before being allowed to complete the following steps

    - If you are not allowed access, please get the enclosure designer to read this document

- Navigate to the desired document (eg: LOW VOLTAGE)

- Select the plus button on the bottom left-hand side of the screen

- Create a folder

- Again select the plus button on the bottom left-hand side of the screen

- Select 'Import'

- Navigate to desired step file on PC

- Click 'Combine to a single Part Studio'

  - Tick:

    - 'Import appearances'

  - Untick:

    - 'Orient imported models with Y Axis Up'

    - 'Join adjacent surfaces'

- Click 'import'

- Double click the imported part, usually 'Import 1's

- Tick 'Create composite'

- Click the green check box

- Wait for it to think

- Ensure the composite part is the name of the given PCB

- Done!

# How To Import PCBs Into an Assembly

- Only complete this section if requested by a member from the mechanical team

- Navigate to the desired Assembly

- Click 'Insert'

- Ensure to click 'Composite parts'

- Search for the desired PCB

- Clicking on the part will bring it into the Assembly