| Activity / Task / Location: Operating the Taylor DE150 dynamometer with NU Racing Accumulator and Emrax 228 Medium Voltage Motor in ECLG02 | Reviewed / Approved By: Signature and Date: |  |
| --- | --- | --- |
| Risk Assessment Developed by: Samuel Connett |  | Date:24/11/2025 |
| This Risk Assessment is to be reviewed every 2 years or whenever changes are required to the materials, equipment or procedure – whichever comes sooner. However, if a safety incident occurs while this RA is being followed, this RA must be reviewed immediately. |  |  |

| Risk Matrix |  | Likelihood |  |  |  |
| --- | --- | --- | --- | --- | --- |
| N.B. For more details regarding use of this matrix / definitions refer to final page of this document | Rare | Unlikely | Possible | Likely | Almost Certain |
| Severe Eg. Potential Fatality or Injury or Illness with permanent disability | MEDIUM | MEDIUM | HIGH | EXTREME | EXTREME |
| Major Eg. Potential Lost Time Injury (but non-permanent disability) | LOW | MEDIUM | MEDIUM | HIGH | EXTREME |
| Moderate Eg. Potential Medical Treatment injury or illness (but no lost time) | LOW | LOW | MEDIUM | MEDIUM | HIGH |
| Minor Eg. Potential First Aid injury | LOW | LOW | LOW | MEDIUM | MEDIUM |
| Minimal Eg. Hazard or near miss requiring reporting and follow up action | LOW | LOW | LOW | LOW | LOW |

Consequence

Actions required based on Risk Assessment

| Extreme | An “extreme” risk requires immediate assessment and senior staff consideration is required; a detailed mitigation plan must be developed, and consideration should be given to ceasing the activity unless the risk can be reduced to a level of high or less; regular monitoring and reported on to the relevant management/steering committee; Target resolution should be within 1 month. |
| --- | --- |
| High | A “high” risk may also require immediate assessment and senior staff consideration; a mitigation plan must be developed; regular monitoring and reported on to the relevant management/steering committee. Target resolution (ideally reduction to medium or low level of risk) should be within 3 months. |
| Medium | A mitigation plan must be developed; existing controls need to be reviewed. Target resolution (ideally reduction to low level of risk) should be within 1 year. |
| Low | Risk is tolerable; manage by well established, routine processes/procedures and be mindful of changes to nature of risks. |

| Hazard Identification and initial Risk Rating |  |  | Control measures and Residual Risk Rating |  | Remaining Hazards | Actions required |
| --- | --- | --- | --- | --- | --- | --- |
| What are the steps of the activity / items of equipment? | What are the potential hazards? | Risk Rating based on Risk Matrix | What control methods or measures will be used to reduce the likelihood and/or the consequence of an illness or injury from those hazards? | Residual Risk Rating based on Risk Matrix | What hazard remains? | What additional actions are required (by who and in what timeframe) to raise the level of control? |
| Dynamometer Set Up | Plugging in and checking cables (trip hazard and LV electrocution) | Medium | Heat shrink insulation on cables, and cables routed safely. Along the floor to the wall and along the walls. | Low | trip hazard and LV electrocution |  |
|  | Checking Motor Mount, driveshaft, driveshaft guard, TS stack and Accumulator (pinch points, heavy things, manual handling) | Medium | These checks are done using at least 2 people. No loose clothing. | Low | Pinch points, heavy things, manual handling |  |
|  | Cooling Water leaking (slips or burns) | High | Anti-slip Steel-cap boots must be worn. If the motor/inverter cooling systems must be checked, then allow 1 hour from the last operation of the motor or inverter before inspecting. | Low | slips or burns |  |
|  | Opening the PAU (HV electrocution, asbestos inhalation) | High | Do not inspect the inside of the PAU without Malcolm’s approval. Do not inspect the PAU with the E-Stop not depressed and mains power disconnected. | Medium | HV electrocution, asbestos inhalation |  |
|  | Opening the E-Stop box (LV electrocution) | Medium | Do not open the E-Stop box unless mains power is disconnected and the power supply is off. | Low |  |  |
| Moving the Accumulator | Using the trolley (trolley rolling back into the user) | Medium | Accumulator trolley brake is checked before leaving TA | Low | trolley rolling back into the user |  |
|  | Carrying the accumulator (pinch points, heavy things, manual handling) | High | Transporting the accumulator requires 4 people. 2 to carry the accumulator. 1 to coordinate, clear the path, and help carry if someone struggles. 1 to transport the accumulator trolley when the accumulator is being carried. | Medium | Pinch points, heavy things, manual handling |  |
|  | Using the Accumulator (HV electrocution) | Medium | The accumulator must not be opened outside of the TA High Voltage Bay (TA105). The HV looms must only be connected after being checked for damage. | Low | HV Electrocution |  |
| Dynamometer Start up | Short Circuit Open Circuit damage to components or people. (LV/HV Electrocution or Circuit Damage) | Medium | Checking cabling before turning on, check safety systems after making any changes | Low | Short Circuit or Open Circuit |  |
|  | Overcharge or undercharge causing cell damage, (cell damage) | Medium | check SoC of battery, ensure accumulator charging in TA while observing the cell voltages on Orion BMS 2 Utility, | Low | Overcharge or undercharge |  |
|  | Faulty, or out-of-place components, or a re-designed component (physical or electrical damage to components) | High | Hand spin driveshaft first, then run motor first without dyno braking and then brake with no motor running, check safety systems after making any changes. Operator outside ECLG02 when the motor or dynamometer are powered with HV. No loose clothing. | Low | Physical or electrical damage to components |  |
| Dynamometer operation | Dynamometer, Motor or Inverter overheating (damage to Dynamometer, Motor or Inverter) | Medium | Make sure dynamometer cooling systems are running before turning on the dynamometer. Make sure motor and inverter cooling systems are running before turning on the motor or inverter. Dynamometer, Motor and Inverter Temperatures monitored on YourDyno. | Low | Dynamometer, Motor or Inverter overheating |  |
|  | Mechanical Driveshaft/Motor/Motor Mount failure (damage to components or nearby people) | High | Emergency procedures in place. Operator and all persons outside of ECLG02. Driveshaft guard must be closed, surrounding the driveshaft. | Low |  |  |
|  | Electrical faults and failures (damage to components, or HV/LV electrocution) | High | Shutdown circuit, Hard Fault Reset board and E-Stops in operation. Cables are insulated. Operator is outside ECLG02. Emergency procedures in place. | Low | Damage to components |  |
|  | Someone entering ECLG02 (HV/LV electrocution, rotating components) | Extreme | Ensure everyone is outside the room. Shut and lock the door to ECLG02 before operating. Operator is outside ECLG02. When someone is approaching, E-Stop and stop the person. | Low |  |  |
|  | Systems outside ECLG02 become energised with high current and/or high voltage (LV/HV electrocution) | Medium | Shutdown circuit will shut down the motor. Loom junction keeps high current paths inside the room. The YourDyno and its USB cannot support HV or high current. | Low |  |  |
|  | Excessive motor RPM (damage to motor and driveshaft) | Medium | Motor RPM is observed using the motors and the dynamometer’s RPM sensors. | Low | Damage to motor and driveshaft |  |
| Shutdown | Electrical systems failing to deenergise (harm to people shutting down the systems) | Medium | Wait for the TSAL to stop flashing red, ensure GLV and TSMS are switched off an E-Stop depressed before entering. | Low |  |  |
|  | Rotating driveshaft (harm to people shutting down the systems) | Medium | Check INV Motor Speed on YourDyno is below 30, torque potentiometer is off, and GLV and TSMS are off before entering | Low |  |  |
|  | Dynamometer, Inverter or Motor overheating (damage to components) | Medium | Dynamometer, Motor and Inverter Temperatures monitored on YourDyno. The dynamometer and inverter/motor cooling systems are turned off after the dynamometer braking is set to 0% and the TSMS is off. | Low |  |  |
| Emergency Shutdown | E-Stop fails during emergency (harm to components and people) | High | Turn off dyno braking, torque potentiometer, GLV, and TSMS, and unplug E-Stop box USB from computer. Also MSD can be disconnected if safe and necessary | Low |  |  |
|  | Building evacuation (harm to people and components) | Extreme | Press E-Stop button. | Medium | Remaining Rotational motion and accumulator (physical or electrical harm to emergency services) |  |
|  | Accumulator battery fire | Medium | E-Stop. Fire Extinguishers near the emergency exit door. Operator outside ECLG02. | Low |  |  |
|  | Electrical fire | Medium | E-Stop. Fire Extinguishers near the emergency exit door. Operator outside ECLG02. | Low |  |  |
|  | Mechanical failure | Medium | Driveshaft guard and motor mount in place. Operator outside ECLG02. E-stop. | Low |  |  |
|  | Cooling systems fail | Medium | E-Stop. | Low |  |  |
|  | Put some detail in this column of thing happening and then damage or pain associated |  |  |  |  |  |

| I have read and understood this Risk Assessment and will follow all associated control measures. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name: |  | Signature: |  | Date: |  |
| Name: |  | Signature: |  | Date: |  |
| Name: |  | Signature: |  | Date: |  |

| Summary of Requirements based on Risk Assessment |  | Review Period / Date |
| --- | --- | --- |
| Personal Protective Equipment | Anti-slip Steel-toe boots, no loose clothing, |  |
| Other Equipment and Equipment Protection | Extinguishers, |  |
| Training Requirements | Induction to EC and TA. |  |
| Procedures, SOPs etc | Operating the Taylor DE150 dynamometer with NU Racing Accumulator and Emrax 228 Medium Voltage Motor in ECLG02 |  |
| Relevant Legislation etc. | WHS Act 2011 (NSW) & Regulations / Codes of Practice |  |

## Questions to ask in order to determine the hazards relating to the task:

| A Could people be injured or made sick by things such as: - Noise - Light - Radiation - Toxicity - Infection - High or low temperatures - Electricity - Moving or falling things (or people) - Flammable or explosive materials - Things under tension or pressure (compressed gas or liquid; springs) - Any other energy sources or stresses - Biohazardous material - Laser | D What could go wrong? - What if equipment is misused? - What might people do that they shouldn’t - How could someone be killed? - How could people be injured? - What may make people ill? - Are there any special emergency procedures required? |
| --- | --- |
|  | E Are procedures or organisational systems missing or not being followed? - Standard Operating Procedures? - Risk Assessments? - Induction or training? - Management of change? - Safety Inspections? - Hazard reporting? - Contractor Management? |
| B Can workplace practices cause injury or sickness? - Are there heavy or awkward lifting jobs? - Can people work in a comfortable posture? - If the work is repetitive, can people take breaks? - Are people properly trained? - Do people follow correct work practices? - Are there adequate facilities for the work being performed? - Are universal safety precautions for biohazards followed? - Is there poor housekeeping? Look out for clutter - Torn or slippery flooring - Sharp objects sticking out - Obstacles | F What kinds of injuries could possibly occur? - Broken bones - Eye damage - Hearing problems - Strains or sprains - Cuts or abrasions - Bruises - Burns - Lung problems including inhalation injury/ infection - Skin contact - Poisoning - Needle-stick injury - Psychological illness or injury |
| C Imagine that a child was to enter your work area? - What would you warn them to be extra careful of? - What would do to reduce the harm to them? |  |

How to Assess Risk

| ## Step 1 – Consider the Consequences What are the potential consequences of an incident occurring? Consider what could reasonably happen as well as what may actually happen. Look at the descriptions and choose the most suitable Consequence. |  | ## Step 2 – Consider the Likelihood What is the likelihood of the consequence identified in step 1 happening? Consider this with the current controls in place. Look at the descriptions and choose the most suitable Likelihood. |  | Step 3 – Calculate the Risk Rating A. Take Step 1 rating and select the correct column. B. Take Step 2 Rating and select the correct line. C. The calculated risk rating is where the two ratings cross on the matrix below. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Consequence |  | Likelihood |  |  |  | ### LIKELIHOOD |  |  |  |  |
|  |  |  |  |  |  | Rare | Unlikely | Possibly | Likely | Almost Certain |
| #### Serious | ##### Potential Fatality or Injury or Illness with permanent disability | Almost Certain | The event could be expected to occur in most circumstances: “This is a common problem here”. | CONSEQUENCE | #### Serious | MEDIUM | MEDIUM | HIGH | EXTREME | EXTREME |
| Major | Potential Lost Time Injury requiring time off work (but non-permanent disability) | Likely | The event has a reasonable chance of occurring in usual conditions: “It has happened here before”. |  | Major | LOW | MEDIUM | MEDIUM | HIGH | EXTREME |
| Moderate | Potential medical treatment Injury or Illness but no lost time | Possible | The event might occur occasionally, has occurred sometime: “Has infrequently happened here before”. |  | Moderate | LOW | LOW | MEDIUM | MEDIUM | HIGH |
| Minor | Potential First Aid Injury | Unlikely | The event has a small chance of occurring. “It has not happened here but has occurred elsewhere”. |  | Minor | LOW | LOW | LOW | MEDIUM | MEDIUM |
| Minimal | No injury but hazard exists or near miss occurred requiring reporting and follow up action | Rare | Very unlikely to occur. “It would be extremely rare for it to occur here”. |  | Minimal | LOW | LOW | LOW | LOW | LOW |

|  | Control Type Eliminate | Example Removing the hazard, eg taking a hazardous piece of equipment out of service. |
| --- | --- | --- |
|  | Substitute | Replacing a hazardous substance or process with a less hazardous one, eg substituting a hazardous substance with a non-hazardous substance. |
|  | Engineering | Redesign a process or piece of equipment to make it less hazardous, Isolating the hazard from the person at risk, eg using a guard or barrier, or containing the hazard in an enclosure. |
|  | Administrative | Adopting safe work practices or providing appropriate training, instruction or information. |
|  | Personal Protective Equipment (PPE) | The use of personal protective equipment could include using gloves, glasses, earmuffs, aprons, safety footwear, dust masks. NOTE: This is a last resort control and should be used in conjunction with higher level controls. |

Controlling the Risk: Risk control is a method of managing the risk with the primary emphasis on controlling the hazards at source. For a risk that is assessed as “extreme” or “high”, steps should be taken immediately to minimize risk of injury.  The method of ensuring that risks are controlled effectively is by using the “hierarchy of controls”.  The Hierarchy of Controls are: